APC mk2 / EOS BRIDGE

APC mk2 / Eos Bridge System

Community Edition  |  v16.0  |  AI-Assisted Development

⚠  AI-Assisted Development

This system was developed with the assistance of Claude (Anthropic AI). It is provided as-is, without warranty of any kind. Always test thoroughly before using in a live show environment and have a backup control method available. Use at your own risk.

Feedback, bug reports and improvements are welcomed. Open an issue or pull request on GitHub.

# 1. System Overview

This system transforms an Akai APC mini mk2 MIDI controller into a full-featured ETC Eos lighting control surface with real-time LED feedback, fader control, and touchscreen configuration. It runs entirely on the QLab Mac mini M4 in the touring rack and requires no additional hardware beyond the existing rig.

The system consists of three files:

| Feature | Description |
| --- | --- |
| eos_apc_feedback.py | The Python bridge script. Runs silently at boot as a background service. Handles all MIDI, OSC, LED feedback, fader control and status reporting. |
| eos_apc_config.json | The configuration file. Defines every button's label, OSC command, LED colour and behaviour. Edit this to customise the surface without touching code. |
| eos_apc_editor.html | The web-based configuration editor. Open in Chrome on the touchscreen to edit buttons, view live connection status, fire OSC commands and reload the bridge. |

## 1.1 Signal Flow

APC mk2 (USB MIDI) → Python Bridge → ETCnomad / QLab (OSC over Ethernet)

ETCnomad (OSC feedback) → Python Bridge → APC mk2 (LED updates)

## 1.2 Hardware Requirements

- Mac mini M4 (QLab Mac) running macOS

- Akai APC mini mk2 connected via USB

- ETCnomad Mac connected via Ethernet on the 2.x.x.x network

- Network switch connecting both Macs

## 1.3 Network Configuration

| QLab Mac IP: | YOUR_BRIDGE_IP (e.g. 2.0.0.2) |
| --- | --- |

| ETCnomad Mac IP: | YOUR_EOS_IP (e.g. 2.0.0.1) |
| --- | --- |

| OSC Listen Port: | 9001 (script receives from Eos) |
| --- | --- |

| OSC Send Port: | 8000 (script sends to Eos) |
| --- | --- |

| QLab OSC Port: | 53000 |
| --- | --- |

| Status HTTP Port: | 9002 (editor polls this) |
| --- | --- |

# 2. Startup & Shutdown

## 2.1 Normal Startup

The bridge starts automatically when the QLab Mac boots. No user action required. On startup:

- All 64 grid buttons sweep through colours (self-test)

- HELLO scrolls across the grid in colour

- All buttons illuminate in their configured colours

- GO button blinks red until Eos connection is confirmed

- GO button turns solid green when Eos responds

## 2.2 Manual Restart

If the bridge needs restarting, use either method:

From the editor — click the green RELOAD button in the top bar. The APC will go dark for approximately 5 seconds then restart with the self-test sequence.

From Terminal:

launchctl unload ~/Library/LaunchAgents/com.eos.apcfeedback.plist && launchctl load ~/Library/LaunchAgents/com.eos.apcfeedback.plist

## 2.3 APC Disconnection / Reconnection

If the APC USB cable is disconnected, the bridge automatically detects the loss within 2 seconds. When the APC is reconnected, the bridge detects it within 2 seconds and automatically:

- Runs the self-test colour sweep

- Plays the HELLO animation

- Restores all button colours and LED states

- Restores fader mode (Sub/ML/Colour)

No manual intervention required.

# 3. Button Layout & LED Colours

## 3.1 Colour Coding

Buttons are colour-coded by function group:

| Feature | Description |
| --- | --- |
| Bright Green | GO button, Enter, Full, Assert, Go To Cue — execute/confirm actions |
| Amber / Orange | General command keys — Cue, Sub, Group, Part, At, Rem-Dim, Out, Undo, Full |
| Cyan / Teal | Utility and palette keys — Copy To, Label, Mark, Macro, Palettes, Trace, Home |
| Light Red | Record operations — Record, Update, Record Only, Time, Delay, Clear |
| Bright Red | Stop button — always prominent |
| White | Numeric keypad — 0-9, +, -, Thru, /, +%, -% |
| Green (right col) | Right column mode keys — Live, Blind, Flexi, Channel, Fixture, Next, Prev, Format |

## 3.2 Button Behaviour

When a button is pressed:

- Button flashes white on press

- Numeric keys flash amber

- Button returns to its idle colour on release

- OSC command fires on press (value 1) and release (value 0) for most keys

- Some keys send empty [ ] instead of 1/0 — configured per button in the editor

## 3.3 GO Button States

| Feature | Description |
| --- | --- |
| Solid green | Ready — Eos connected, no cue running |
| Fast green blink | Cue actively running |
| Fast red blink | Eos offline — connection lost |

## 3.4 Right Column (Bottom Row in Editor)

The right column buttons on the physical APC (Live, Blind, Flexi, Channel, Fixture, Next, Prev, Format) appear as the bottom row in the web editor. They light up green and are single-colour LEDs.

- Blind button blinks while Blind mode is active in Eos

- Live button blinks while Live mode is active

# 4. Fader Control

## 4.1 Fader Modes

The 9 faders operate in three modes, switched by the track buttons above faders 7 and 8:

| Feature | Description |
| --- | --- |
| Submaster Mode (default) | Faders 1-8 control Eos submasters 1-8 directly. Fader 9 controls the Grandmaster. Active when no mode button is lit. |
| ML Encoder Mode | Faders 1-5 control moving light parameters — Intensity, Pan, Tilt, Zoom, Edge. Toggle with button above fader 8 (blinks red when active). Pan and Tilt use Switch method — move fader to set speed and direction, press the track button above that fader to stop movement. |
| Colour Mode | Faders 1-8 control colour mixing parameters — Red, Lime, Green, Blue, Hue, Saturation. Toggle with button above fader 7 (blinks red when active). |

## 4.2 ML Encoder Pan/Tilt Control

Pan and Tilt use a continuous Switch method rather than absolute positioning:

- Push fader up = move in positive direction at speed proportional to fader position

- Push fader down = move in negative direction

- Press the track button above the fader = STOP movement immediately

- The track button blinks while movement is active

Pan and Tilt cannot be set to an absolute position with the faders. Use the Eos encoder wheels for precise positioning, then use faders for live adjustments.

# 5. Configuration Editor

## 5.1 Opening the Editor

Open eos_apc_editor.html in Chrome on the QLab Mac touchscreen. The file is located at:

~/Documents/eos_apc_editor.html

## 5.2 Status Bar

The top bar shows live system status updated every 2 seconds:

| Feature | Description |
| --- | --- |
| EOS indicator | Green = connected and responding. Red blinking = offline or not responding. |
| QLAB indicator | Green = reachable on the network. Amber = not responding (check QLab is running). |
| BRIDGE indicator | Green = Python script running. Red = script crashed or not started. |
| FADERS mode | Shows current fader mode — SUB, ML or COLOR highlighted. |
| File indicator | Shows which config file is loaded. |

## 5.3 Loading the Config

The editor cannot load files automatically due to browser security. To load:

- Click LOAD JSON in the top bar

- Navigate to ~/Documents/ and select eos_apc_config.json

- The grid will populate with all button labels and colours

The editor remembers the last loaded filename and shows it in the status bar as a reminder.

## 5.4 Editing a Button

- Tap any button on the grid = fires its OSC command immediately

- Long press (hold 400ms) = opens the button in the editor panel on the right

The editor panel shows:

- Label — the display name shown on the grid

- OSC Command — the full OSC address sent to Eos or QLab

- Target — eos or qlab

- Group — affects LED colour behaviour on press (go/command/numeric/destructive/shift)

- Trigger — Send 1/0 fires the key on press and releases on lift. Send [] fires a momentary trigger with no value.

- LED Colour — pick from palette swatches or enter a custom velocity value (0-127)

## 5.5 Saving Changes

The workflow for saving config changes:

- Make changes in the editor

- Click SAVE JSON — downloads eos_apc_config.json to ~/Downloads/

- In Terminal, type: updateconfig

- Click RELOAD in the editor — APC restarts in ~5 seconds

The updateconfig command copies ~/Downloads/eos_apc_config.json to ~/Documents/eos_apc_config.json automatically.

## 5.6 The RELOAD Button

The green RELOAD button in the top bar restarts the bridge script. Use this after:

- Copying a new config file to ~/Documents/

- Any time the APC behaves unexpectedly

- After updating eos_apc_feedback.py

The APC will go dark for approximately 5 seconds during reload then run the self-test and HELLO animation.

# 6. OSC Reference

## 6.1 Key Commands

All Eos key commands follow the pattern /eos/key/keyname where keyname is the lowercase OSC name visible in the Eos virtual keyboard under the OSC /key Names dropdown.

Important confirmed key names in this rig:

| Feature | Description |
| --- | --- |
| /eos/key/go_0 | GO — fires cue on cue list 0 |
| /eos/key/stop | Stop |
| /eos/key/flexichannel_mode | Flexi (note: no underscore between flexi and channel) |
| /eos/key/format | Format (send [] not 1/0) |
| /eos/key/fixture | Fixture (send [] not 1/0) |
| /eos/key/blind | Blind |
| /eos/key/live | Live |
| /eos/key/chan | Channel (not channel) |
| /eos/key/\\ | Forward slash / key |
| /eos/key/clear_cmd | Clear (NOT /eos/key/clear which clears show data) |

## 6.2 Fader OSC Commands

| Feature | Description |
| --- | --- |
| /eos/sub/N | Set submaster N level. Value 0.0 to 1.0. N = 1 to 8. |
| /eos/grandmaster | Set grandmaster level. Value 0.0 to 1.0. |
| /eos/param/intens | Set intensity on selected fixtures. Value 0.0 to 100.0. |
| /eos/param/pan | Set pan on selected fixtures. Value 0.0 to 100.0 (percent of range). |
| /eos/param/tilt | Set tilt on selected fixtures. Value 0.0 to 100.0. |
| /eos/switch/pan | Continuous pan movement. Positive value = positive direction, negative = negative. Send 0 to stop. |
| /eos/switch/tilt | Continuous tilt movement. |

## 6.3 Eos Feedback OSC

Eos sends these OSC messages back to the bridge (ETCnomad Setup > System > Show Control > OSC TX must point to your bridge Mac IP on port 9001):

| Feature | Description |
| --- | --- |
| /eos/out/active/cue | Current cue progress. Value 0.0 to 1.0. 0 = no cue, 1.0 = cue complete. |
| /eos/out/blind | Blind mode state. 1 = active, 0 = inactive. |
| /eos/out/live | Live mode state. |
| /eos/out/ping | Response to /eos/ping. Used for connection monitoring. |

# 7. Complete Feature List

| Feature | Description |
| --- | --- |
| RGB LED feedback | All 64 grid buttons have individually addressable RGB LEDs configured from the editor |
| Colour-coded layout | Buttons colour-coded by function group for fast visual identification in any lighting condition |
| Startup self-test | All LEDs sweep through colours on boot to confirm all pads are working |
| HELLO animation | HELLO scrolls across the grid in colour after self-test |
| GO cue feedback | GO button blinks fast green while a cue is actively running |
| Cue complete | GO button returns to solid green when cue reaches 100% |
| Eos connection indicator | GO button blinks red if Eos connection is lost |
| Blind mode feedback | Blind button on right column blinks while Eos is in Blind mode |
| Live mode feedback | Live button on right column blinks while Eos is in Live mode |
| Button press flash | All buttons flash white on press and return to idle colour on release |
| Numeric key highlight | Numeric keys flash amber instead of white for distinction |
| Submaster faders | Faders 1-8 control Eos submasters 1-8. Value 0-100%. |
| Grandmaster fader | Fader 9 controls the Eos grandmaster |
| ML encoder faders | Faders 1-5 control Intensity, Pan, Tilt, Zoom, Edge on selected fixtures |
| Pan/Tilt Switch method | Pan and Tilt use continuous movement control with track button stop |
| Colour faders | Faders 1-8 control colour mix parameters — Red, Lime, Green, Blue, Hue, Saturation |
| Three fader modes | Sub, ML and Colour modes toggled by track buttons above faders 7 and 8 |
| Mode LED indicators | Track buttons blink red to show which fader mode is active |
| QLab GO button | Dedicated button fires QLab GO via OSC directly to the QLab Mac |
| Right column keys | Live, Blind, Flexi, Channel, Fixture, Next, Prev, Format all functional |
| Auto-reconnect | If APC USB disconnects, bridge detects it in 2 seconds and reconnects automatically when cable is restored |
| Auto-restore | All LED states and fader mode restored automatically on reconnect |
| Auto-start on boot | Launch agent starts bridge automatically on Mac login, restarts if crashed |
| Web config editor | Chrome-based editor shows real grid layout with button colours |
| Live status bar | Editor shows Eos, QLab, Bridge connection status updated every 2 seconds |
| Fader mode display | Editor shows current fader mode in real time |
| Touchscreen fire | Tap any button in editor to fire its OSC command immediately |
| Long press edit | Long press any button in editor to open its settings |
| FIRE button | Each button's editor panel has a dedicated fire button for testing |
| RELOAD button | Editor has one-tap bridge restart without Terminal |
| updateconfig alias | Terminal command to deploy saved config in one step |
| Logging | All OSC commands, MIDI events and connection changes logged to ~/Documents/eos_apc.log |
| Config-driven | All button mappings, colours, OSC commands and network settings in JSON — no code editing needed |
| Status API | HTTP server on port 9002 exposes live status JSON for the editor |
| Web fire API | HTTP POST /fire endpoint allows editor to fire OSC commands through the bridge |
| CPU usage | Less than 0.1% CPU at idle on Apple Silicon Mac mini |

# 8. Troubleshooting

## 8.1 Quick Diagnosis

Check the log first for any error:

tail -20 ~/Documents/eos_apc.log

Check if the script is running:

launchctl list | grep apcfeedback

A number in the first column means it is running. A dash means it crashed.

Check if the APC is visible to the system:

~/scripts/venv/bin/python3 -c "import mido; print(mido.get_output_names())"

## 8.2 Troubleshooting Table

| Symptom | Cause | Fix |
| --- | --- | --- |
| APC is dark, no LEDs | Script not running or crashed | Check launchctl list ¦ grep apcfeedback. If dash, run: launchctl load ~/Library/LaunchAgents/com.eos.apcfeedback.plist |
| APC dark after USB unplug | MIDI port lost | Reconnect USB. Bridge auto-reconnects in 2 seconds. |
| GO button blinking red | Eos connection lost | Check ETCnomad is running and OSC TX is pointing to your bridge Mac IP on port 9001 |
| Buttons not sending to Eos | Wrong OSC command or port | Check eos_apc_config.json OSC commands. Check Eos OSC RX is enabled on port 8000. |
| SyntaxError in log | Smart quotes corrupted script | Run the python3 byte replacement command to clean smart quotes, then reload |
| Bridge OFFLINE in editor | Script crashed or HTTP server not started | Reload bridge from Terminal. Check log for error. |
| RELOAD button takes bridge offline | launchctl unload killing the process before reload fires | This was fixed in v16 with start_new_session=True. If it recurs, reload manually from Terminal. |
| Faders not controlling subs | Wrong fader mode active | Check editor status bar — should show SUB highlighted. Press button above fader 8 to toggle back. |
| Pan/tilt not stopping | Switch still running | Press track button above the relevant fader to stop movement. |
| Config changes not taking effect | Wrong JSON file in Documents | Run updateconfig in Terminal then press RELOAD in editor. |
| Editor shows wrong colours | JSON not loaded | Click LOAD JSON and select ~/Documents/eos_apc_config.json |
| launchctl load failed: I/O error | Plist file corrupted | Delete and recreate plist file. See plist recreation command in setup notes. |
| Flexi not working | Wrong OSC key name | Correct name is /eos/key/flexichannel_mode (no underscore between flexi and channel) |
| Format not working | Sending value instead of empty | Format requires Send [] not Send 1/0. Check use_val setting in editor. |
| QLab GO not firing | Wrong IP or port | QLab must have OSC enabled on port 53000. Check Network settings in editor. |

## 8.3 The Smart Quote Problem

macOS automatically converts straight quotes to curly/smart quotes when files are downloaded or opened in certain applications. Python cannot parse smart quotes, causing SyntaxError: invalid character.

Prevention: Always write Python script updates using the cat > file << 'ENDOFSCRIPT' method in Terminal. Never download and copy .py files from a browser.

Fix (run in Terminal if script fails with quote errors):

python3 << 'PYEOF'

content = open('/Users/YOUR_USERNAME/Documents/eos_apc_feedback.py', 'rb').read()

content = content.replace(b'\xe2\x80\x9c', b'"').replace(b'\xe2\x80\x9d', b'"')

content = content.replace(b'\xe2\x80\x98', b"'").replace(b'\xe2\x80\x99', b"'")

open('/Users/YOUR_USERNAME/Documents/eos_apc_feedback.py', 'wb').write(content)

print('Fixed')

PYEOF

## 8.4 Recreating the Launch Agent

If the plist file is corrupted (Load failed: 5 error):

rm ~/Library/LaunchAgents/com.eos.apcfeedback.plist

Then recreate with the cat > plist << 'ENDPLIST' method using the full plist XML. See original setup notes.

# 9. File & Command Reference

## 9.1 File Locations

| Feature | Description |
| --- | --- |
| ~/Documents/eos_apc_feedback.py | Main bridge script |
| ~/Documents/eos_apc_config.json | Active configuration file |
| ~/Documents/eos_apc_editor.html | Web configuration editor |
| ~/Documents/eos_apc.log | Runtime log — all events and errors |
| ~/Library/LaunchAgents/com.eos.apcfeedback.plist | Launch agent — auto-start on boot |
| ~/scripts/venv/ | Python virtual environment with all dependencies |

## 9.2 Terminal Commands

| Feature | Description |
| --- | --- |
| updateconfig | Copy downloaded config from Downloads to Documents |
| tail -f ~/Documents/eos_apc.log | Watch log in real time (Ctrl+C to stop) |
| launchctl list ¦ grep apcfeedback | Check if bridge is running |
| launchctl load ~/Library/LaunchAgents/com.eos.apcfeedback.plist | Start bridge |
| launchctl unload ~/Library/LaunchAgents/com.eos.apcfeedback.plist | Stop bridge |
| pkill -f eos_apc_feedback.py | Force kill bridge process |
| ~/scripts/venv/bin/python3 -c "import mido; print(mido.get_output_names())" | List MIDI ports visible to the bridge |

## 9.3 Python Dependencies

Installed in ~/scripts/venv/ via pip:

- python-osc — OSC send and receive

- mido — MIDI message handling

- python-rtmidi — MIDI port access

## 9.4 ETCnomad OSC Setup

In ETCnomad: Setup > System > Show Control > OSC

| Feature | Description |
| --- | --- |
| OSC RX Enabled | Yes |
| OSC RX Port | 8000 |
| OSC TX Enabled | Yes |
| OSC TX IP | YOUR_BRIDGE_IP — the IP of the Mac running the bridge |
| OSC TX Port | 9001 |

# 10. Version History

| Feature | Description |
| --- | --- |
| v16.0 | Auto-reconnect watchdog. APC reconnects automatically on USB restore. Thread-safe MIDI writes. |
| v15.0 | Config-driven from JSON. Web editor with status bar. RELOAD button. /fire API endpoint. /reload endpoint with start_new_session fix. |
| v14.0 | Right column note map confirmed from log. HELLO animation. Startup self-test. |
| v13.0 | OSC/PILOT eliminated. Python sends all OSC directly. Shift key. Macro shortcuts (later removed). |
| v12.0 | Switch method for pan/tilt. Colour fader mode. Three fader modes. Effects mode (later removed). |
| v11.0 | Format and Fixture key investigation. Right column debug logging. |
| v10.0 | Ping-based Eos connection monitoring. GO blinks red on disconnect. |
| v1-9 | Initial development — LED feedback, OSC bridge, fader modes, colour coding. |

# 11. Installing on a Different Rig

This section covers everything needed to install the APC mk2 / Eos bridge on a new Mac. Two methods are available: the interactive installer script which handles everything automatically, or a step-by-step manual process.

## 11.1 Requirements

Hardware:

- Mac running macOS (Intel or Apple Silicon)

- Akai APC mini mk2 connected via USB

- ETC Eos family console or ETCnomad on a second machine, connected via Ethernet

- Both machines on the same network (recommended: 2.x.x.x / 255.0.0.0 static IPs)

Software:

- Python 3 (comes with macOS or install from python.org)

- Homebrew (optional but recommended for Python management)

- Chrome or Safari for the web editor

## 11.2 Method A — Automated Installer (Recommended)

The installer script install_eos_apc.sh handles all steps automatically. It asks a series of questions about your network and preferences, then installs everything without any manual file editing.

What the installer does:

- Detects the Mac username and home directory

- Asks for all network settings — bridge IP, Eos IP, ports, QLab IP

- Creates the Python virtual environment and installs all dependencies

- Writes eos_apc_config.json with your network settings pre-filled

- Writes eos_apc_feedback.py using Python directly — bypassing the smart quote problem entirely

- Verifies the Python syntax is clean before proceeding

- Copies eos_apc_editor.html from the same folder as the installer

- Creates and loads the launch agent for auto-start

- Adds the updateconfig alias to .zshrc or .bash_profile

- Prints a final checklist including the exact ETCnomad OSC settings to enter

## 11.3 Running the Installer

Place these three files in the same folder on the new Mac:

- install_eos_apc.sh

- eos_apc_editor.html

- eos_apc_manual.docx (optional — for reference)

Open Terminal, navigate to the folder and run:

chmod +x install_eos_apc.sh

./install_eos_apc.sh

The installer will ask the following questions in order:

| Feature | Description |
| --- | --- |
| Username confirmation | The installer detects your username automatically. Confirm or correct it. |
| Bridge IP | The IP address of this Mac (the one running the bridge). Typically on the 2.x.x.x range for ETC rigs. |
| Eos IP | The IP address of the ETCnomad / Eos Mac. Typically on the 2.x.x.x range for ETC rigs. |
| Eos OSC receive port | The port Eos listens on for incoming OSC. Default: 8000 |
| Bridge listen port | The port the bridge listens on for Eos feedback. Default: 9001 |
| QLab on same Mac? | If yes, QLab IP is set to 127.0.0.1 automatically. If no, you enter a separate IP. |
| QLab OSC port | The port QLab listens on. Default: 53000 |
| Install directory | Where the files are placed. Default: ~/Documents |
| Venv location | Where the Python environment is created. Default: ~/scripts |
| Auto-start on login | Creates and loads the launch agent. Recommended: yes |
| updateconfig alias | Adds the one-word deploy command to your shell. Recommended: yes |

After confirming the summary, the installer runs all steps and ends with a completion screen showing the ETCnomad OSC settings to configure.

The entire install takes approximately 5 minutes on a fast internet connection. The Python package download is the longest step.

## 11.4 After the Installer Completes

The installer will display the exact settings to enter in ETCnomad. In ETCnomad:

- Setup > System > Show Control > OSC

- Set OSC TX IP to the bridge Mac IP you entered

- Set OSC TX Port to the listen port you entered (default 9001)

- Set OSC RX Port to the Eos port you entered (default 8000)

- Set both RX and TX Enabled to Yes

Then connect the APC via USB. It should immediately run the self-test colour sweep and HELLO animation.

## 11.5 Method B — Manual Installation

If you prefer to install manually or need to customise the process, follow these steps. Allow approximately 30 minutes.

On the Mac running the bridge:

- System Settings > Network > Ethernet > Details > TCP/IP

- Set Configure IPv4 to Manually

- IP Address: your chosen static IP (e.g. 2.0.0.2 — ETC standard range)

- Subnet Mask: 255.0.0.0

- Router: leave blank or set to your switch IP

On the ETCnomad Mac:

- Same process — set a static IP on the ETC network range (e.g. 2.0.0.1)

The 2.x.x.x range is the ETC standard for Art-Net and sACN. Using this range avoids conflicts with standard network infrastructure and is recognised by Eos automatically.

### 11.5.1 Step 1 — Install Python and Virtual Environment

Open Terminal and check if Python 3 is installed:

python3 --version

If not installed, install via Homebrew:

/bin/bash -c "$(curl -fsSL https://raw.githubusercontent.com/Homebrew/install/HEAD/install.sh)"

brew install python3

Create the virtual environment and install dependencies:

mkdir -p ~/scripts

python3 -m venv ~/scripts/venv

~/scripts/venv/bin/pip install python-osc mido python-rtmidi

Verify the install:

~/scripts/venv/bin/python3 -c "import mido; print(mido.get_output_names())"

The APC mini mk2 should appear in the list if connected via USB.

### 11.5.2 Step 2 — Copy the Three Files

Place all three files in the Documents folder:

~/Documents/eos_apc_feedback.py

~/Documents/eos_apc_config.json

~/Documents/eos_apc_editor.html

Never copy eos_apc_feedback.py by downloading from a browser and opening in a text editor. macOS will corrupt the quotes. Always write it directly using the cat > file << 'ENDOFSCRIPT' method in Terminal. See the Smart Quote Problem section.

The safest way to deploy eos_apc_feedback.py on a new machine is to copy the full cat command from this manual and paste it into Terminal. The script will be written with guaranteed clean ASCII characters.

The JSON config and HTML editor files are safe to copy normally — they do not contain Python syntax that macOS can corrupt.

### 11.5.3 Step 3 — Edit Network Settings in the Config

Open eos_apc_config.json in a plain text editor (TextEdit in plain text mode, or nano in Terminal) and update the network section to match your rig:

nano ~/Documents/eos_apc_config.json

Find the network section and update:

{

"network": {

"eos_ip": "YOUR_EOS_IP",     <- IP of the Eos/ETCnomad Mac

"eos_osc_port": 8000,         <- Eos OSC receive port

"qlab_ip": "YOUR_BRIDGE_IP", <- IP of the Mac running the bridge

"qlab_osc_port": 53000,       <- QLab OSC receive port

"listen_port": 9001           <- Port bridge listens on for Eos feedback

}

}

Save the file with Ctrl+O then Ctrl+X (in nano).

### 11.5.4 Step 4 — Configure ETCnomad OSC

In ETCnomad (or on the Eos console):

- Go to Setup > System > Show Control > OSC

- Set OSC RX Enabled to Yes

- Set OSC RX Port to 8000

- Set OSC TX Enabled to Yes

- Set OSC TX IP to the IP of the Mac running the bridge

- Set OSC TX Port to 9001

- Save settings

Test connectivity by pressing a key on the Eos virtual keyboard while watching the bridge log. You should see OSC messages arriving.

### 11.5.5 Step 5 — Test the Script Manually

Before setting up auto-start, test the script runs correctly:

~/scripts/venv/bin/python3 ~/Documents/eos_apc_feedback.py

You should see:

- The APC runs the colour sweep self-test

- HELLO scrolls across the grid

- All buttons illuminate in their configured colours

- The log shows Bridge v16.0 starting, then Ready

- GO button blinks red until Eos responds, then turns solid green

Press Ctrl+C to stop. If there are errors, check the Troubleshooting section.

### 11.5.6 Step 6 — Create the Launch Agent

This makes the bridge start automatically at login and restart if it crashes. Run in Terminal — replace USERNAME with your Mac username:

cat > ~/Library/LaunchAgents/com.eos.apcfeedback.plist << 'ENDPLIST'

<?xml version="1.0" encoding="UTF-8"?>

<!DOCTYPE plist PUBLIC "-//Apple//DTD PLIST 1.0//EN"

"http://www.apple.com/DTDs/PropertyList-1.0.dtd">

<plist version="1.0">

<dict>

<key>Label</key>

<string>com.eos.apcfeedback</string>

<key>ProgramArguments</key>

<array>

<string>/Users/USERNAME/scripts/venv/bin/python3</string>

<string>/Users/USERNAME/Documents/eos_apc_feedback.py</string>

</array>

<key>RunAtLoad</key>

<true/>

<key>KeepAlive</key>

<true/>

<key>StandardOutPath</key>

<string>/Users/USERNAME/Documents/eos_apc.log</string>

<key>StandardErrorPath</key>

<string>/Users/USERNAME/Documents/eos_apc.log</string>

</dict>

</plist>

ENDPLIST

Then load it immediately:

launchctl load ~/Library/LaunchAgents/com.eos.apcfeedback.plist

Verify it started:

launchctl list | grep apcfeedback

A number in the first column means it is running successfully.

### 11.5.7 Step 7 — Add the updateconfig Alias

This adds a shortcut command to deploy config changes quickly:

echo 'alias updateconfig="cp ~/Downloads/eos_apc_config.json ~/Documents/eos_apc_config.json && echo Config updated"' >> ~/.zshrc && source ~/.zshrc

If the Mac uses bash instead of zsh (older macOS), replace ~/.zshrc with ~/.bash_profile.

### 11.5.8 Step 8 — Verify Everything

Run through this checklist:

| Feature | Description |
| --- | --- |
| APC self-test runs on boot | Unplug and replug APC USB. Bridge should detect reconnect in 2 seconds and run self-test. |
| All buttons lit | All 64 grid buttons should illuminate in their configured colours. |
| GO button green | GO should be solid green when Eos is running and connected. |
| Buttons fire Eos commands | Press Cue, Enter etc. and verify Eos responds. |
| Faders control submasters | Move faders and confirm sub levels change in Eos. |
| Editor status bar green | Open editor in Chrome. All three status pills should show green. |
| Reload button works | Click RELOAD in editor. APC should restart in ~5 seconds. |
| updateconfig works | Save config from editor, type updateconfig in Terminal. |
| Auto-start on reboot | Restart the Mac. APC should light up automatically after login. |

## 11.6 Adapting for a Different Username

The plist file contains hardcoded paths with the username. On a different Mac, replace all instances of the username with the new Mac username. The easiest way:

NEWUSER="newusername"

sed -i '' "s/YOUR_OLD_USERNAME/$NEWUSER/g" ~/Library/LaunchAgents/com.eos.apcfeedback.plist

## 11.7 Adapting for a Different Network

If the new rig uses a different IP range (e.g. 10.x.x.x or 192.168.x.x):

- Update the network section in eos_apc_config.json with the new IPs

- Update ETCnomad OSC TX IP to match the bridge Mac

- Ensure both Macs are on the same subnet

- The listen_port (9001) and eos_osc_port (8000) rarely need changing

The 2.x.x.x network is strongly recommended for ETC equipment compatibility. Art-Net and sACN both default to this range and Eos auto-discovers nodes on it.

## 11.8 Adapting for a Different APC Model

This system is specifically built for the Akai APC mini mk2. The original APC mini (mk1) uses a completely different LED protocol:

- mk1 uses velocity values 0-6 for colours (Off, Green, Green Blink, Red, Red Blink, Yellow, Yellow Blink)

- mk1 uses MIDI channel 1 only — no channel-based blink modes

- mk1 grid note numbers count from bottom-left upward

- mk2 uses a 128-colour RGB palette controlled by velocity, with MIDI channel controlling brightness/blink behaviour

To adapt for mk1, the send_led and send_peripheral_led functions would need rewriting, and all color constants would need replacing with mk1 velocity values. The button note map would also need remapping.

## 11.9 Adapting for a Different Console

The OSC key names are specific to ETC Eos family software. For other consoles:

- MA2/MA3 — use /gma2/cmd or /gma3/cmd OSC addresses

- Chamsys MagicQ — uses /pb/ and /exec/ OSC namespaces

- Luminex Araxa / other — consult that console's OSC documentation

The fader submaster OSC (/eos/sub/N) and feedback addresses (/eos/out/active/cue etc.) are Eos-specific. These would need replacing with the target console's equivalent addresses. The button grid, LED system, fader modes and editor are all console-agnostic and would work unchanged.

## 11.10 Minimum Install (No Fader Control, No Editor)

For a stripped-down install with only button-to-OSC and LED feedback:

- Follow Steps 1-3 and 6-8 only

- The config JSON is still required for button mappings

- The editor HTML is optional — buttons can be configured by editing the JSON directly in nano

- CPU usage remains under 0.1%

# 12. iPad App (Progressive Web App)

The configuration editor can be installed as a native-feeling app on an iPad or iPhone using Progressive Web App technology. It installs directly from Safari with no App Store required and no Apple Developer account needed.

## 12.1 What the iPad App Offers

| Feature | Description |
| --- | --- |
| Full screen | Runs without browser chrome — no address bar, no Safari UI. Looks and feels like a native app. |
| Home screen icon | Installs to the iPad home screen with the APC EOS icon. Launches instantly. |
| Three-tab layout | Grid tab, Settings tab, Status tab — optimised for touchscreen navigation. |
| Portrait mode | Button grid fills the screen. Tapping a button to edit slides up an editor panel from the bottom. |
| Landscape mode | Grid on the left, editor panel docked on the right — same layout as the Mac desktop editor. |
| Status page | Large easy-to-read status cards for Eos, QLab, Bridge and APC. Visible from across a room. |
| Activity log | Live feed of the last 20 OSC commands fired — useful for monitoring during a show. |
| Offline capable | Service worker caches the app after first load. Opens without internet. |
| Larger touch targets | All buttons are finger-sized — 52px tall grid buttons, large input fields. |
| Tap to fire / hold to edit | Tap fires OSC immediately. Hold 500ms opens the button editor. |

## 12.2 Files Required

The iPad app consists of five files that must all be served from the same folder:

| Feature | Description |
| --- | --- |
| eos_apc_editor.html | The main app HTML — iPad-optimised with tab navigation and slide-up editor sheet |
| manifest.json | PWA manifest — defines app name, icon, colours and display mode |
| sw.js | Service worker — handles caching for offline use and intercepts network requests |
| icon-192.png | App icon 192x192 — shown on home screen and in app switcher |
| icon-512.png | App icon 512x512 — used for splash screen and high-resolution displays |

## 12.3 Automated Installation

The install_ipad_pwa.sh script handles the complete setup automatically. It:

- Checks all five PWA files are present in the same folder as the installer

- Asks for the install directory and HTTP server port

- Copies all five files to the install directory

- Creates a launch agent that starts the HTTP server automatically on boot

- Adds a serveapp alias for manually starting the server from Terminal

- Tests the server is responding and prints the HTTP 200 confirmation

- Prints the exact Safari URL and step-by-step iPad installation instructions

To run the installer, place install_ipad_pwa.sh in the same folder as the five PWA files and run:

chmod +x install_ipad_pwa.sh

./install_ipad_pwa.sh

The installer will ask:

| Feature | Description |
| --- | --- |
| Serve port | The port the HTTP server will listen on. Default: 8080 |
| Install directory | Where the PWA files are placed. Default: ~/Documents/apc-pwa |
| Auto-start server | Whether to create a launch agent that starts the server on boot. Recommended: yes |

## 12.4 Manual Installation

If you prefer to set up manually:

### 12.4.1 Step 1 — Place the files

Copy all five PWA files into a folder on the QLab Mac. The recommended location is:

~/Documents/apc-pwa/

### 12.4.2 Step 2 — Start the HTTP server

The PWA requires HTTP — it cannot run from the filesystem on iOS. Start a simple server:

cd ~/Documents/apc-pwa && python3 -m http.server 8080

Leave this Terminal window open. The server runs on port 8080 and serves the app to any device on the same network.

The server only needs to run when you want to use the iPad app or install it. Once installed on the iPad the app runs from cache even without the server, but live status polling and OSC firing require the Python bridge to be running.

### 12.4.3 Step 3 — Open in Safari on the iPad

On the iPad, open Safari and navigate to:

http://YOUR_BRIDGE_IP:8080/eos_apc_editor.html

Replace YOUR_BRIDGE_IP with the actual IP of the Mac running the bridge. The app will load in Safari.

### 12.4.4 Step 4 — Add to Home Screen

- Tap the Share button (box with upward arrow) in Safari

- Scroll down and tap Add to Home Screen

- Name it APC EOS

- Tap Add in the top right

The app icon appears on the iPad home screen. Tap it to open full screen.

For the best experience, rotate the iPad to landscape before adding to home screen. The app remembers the orientation it was installed in as its default.

### 12.4.5 Step 5 — Make the server auto-start

To avoid manually starting the server before each use, create a launch agent. Replace USERNAME with your Mac username:

cat > ~/Library/LaunchAgents/com.eos.apcpwa.plist << 'ENDPLIST'

<?xml version="1.0" encoding="UTF-8"?>

<!DOCTYPE plist PUBLIC "-//Apple//DTD PLIST 1.0//EN"

"http://www.apple.com/DTDs/PropertyList-1.0.dtd">

<plist version="1.0">

<dict>

<key>Label</key>

<string>com.eos.apcpwa</string>

<key>ProgramArguments</key>

<array>

<string>/usr/bin/python3</string>

<string>-m</string>

<string>http.server</string>

<string>8080</string>

</array>

<key>WorkingDirectory</key>

<string>/Users/USERNAME/Documents/apc-pwa</string>

<key>RunAtLoad</key>

<true/>

<key>KeepAlive</key>

<true/>

</dict>

</plist>

ENDPLIST

launchctl load ~/Library/LaunchAgents/com.eos.apcpwa.plist

## 12.5 Using the App

### 12.5.1 Grid Tab

Shows the full 8x8 button grid plus the right column and track buttons. The grid fills the screen in portrait, or shares space with the editor panel in landscape.

- Tap a button = fires its OSC command immediately

- Hold a button (500ms) = opens it in the editor panel

- A green flash confirms a successful fire

- The bottom row shows the right column buttons (Live, Blind, Flexi etc.)

### 12.5.2 Settings Tab

Network settings, ML fader parameters and colour mode fader assignments. Changes take effect immediately in memory but must be saved via the SAVE button and deployed with updateconfig + RELOAD to persist.

### 12.5.3 Status Tab

Large status cards showing:

- Eos connection — green if connected, red blinking if offline

- QLab connection — green if reachable

- Python bridge — green if the script is running

- APC mk2 — green if connected via USB

- Current fader mode

- Recent activity log — last 20 OSC commands fired

## 12.6 Loading the Config on the iPad

The browser security model on iOS prevents automatic file access. To load the config:

- Tap LOAD in the top bar

- Navigate to the config file — it may be in Files > On My iPad > Downloads, or in iCloud Drive if you transferred it there

- The grid populates with all button labels and colours

The easiest way to get the config onto the iPad is to use AirDrop. On the Mac, right-click eos_apc_config.json and choose Share > AirDrop, then send it to the iPad. It will appear in the Files app.

## 12.7 Network Requirements

The iPad must be on the same WiFi network as the QLab Mac for the app to function. The bridge communicates with the app on port 9002 (status API) and the app polls this every 2 seconds.

If the rig uses a dedicated Ethernet network (2.x.x.x) for lighting control, connect the iPad to a WiFi access point that bridges to that network, or use the Mac's WiFi sharing to create a hotspot on the lighting network.

Do not connect the iPad to a public WiFi network while using it as a show controller. Use a private dedicated network for the rig.

## 12.8 Differences from the Desktop Editor

| Feature | Description |
| --- | --- |
| Tab navigation | Three tabs replace the two-panel desktop layout. Grid, Settings and Status are separate screens on iPad. |
| Editor sheet | In portrait mode the button editor slides up from the bottom as a sheet rather than appearing in a side panel. |
| Landscape layout | In landscape the editor panel docks to the right of the grid — identical to the desktop layout. |
| Button size | Grid buttons are 52px tall on iPad vs 44px on the Mac editor — easier to tap accurately. |
| Font size | Input fields use 15px font for finger-friendly editing vs 13px on desktop. |
| LOAD button label | Shows as LOAD instead of LOAD JSON to save space in the status bar. |
| Status page | The iPad has a dedicated full-screen status page not present in the desktop editor. |
| Activity log | The iPad shows a running log of the last 20 OSC events fired from the app. |

# 13. GitHub Repository

All project files are stored in a private GitHub repository at:

https://github.com/YOUR_USERNAME/APC_MK2-EOS-Bridge

GitHub stores files as binary — no text processing, no quote substitution. This is the safest way to transfer files between Macs and eliminates the smart quote corruption problem entirely.

## 13.1 Repository Contents

| Feature | Description |
| --- | --- |
| eos_apc_feedback.py | Main Python bridge script — v16.0 |
| eos_apc_config.json | Button layout and network configuration |
| eos_apc_editor.html | Desktop web configuration editor |
| APC-EOS_bridge_installer.sh | Interactive installer for new Mac setups |
| apc-pwa/eos_apc_editor.html | iPad PWA app |
| apc-pwa/manifest.json | PWA manifest |
| apc-pwa/sw.js | PWA service worker |
| apc-pwa/icon-192.png | App icon 192x192 |
| apc-pwa/icon-512.png | App icon 512x512 |
| install_ipad_pwa.sh | iPad PWA installer script |

## 13.2 Cloning onto a New Mac

To install the complete system on a new Mac from GitHub:

Step 1 — Install Git if not already installed:

xcode-select --install

Step 2 — Clone the repository:

cd ~/Documents

git clone https://github.com/YOUR_USERNAME/APC_MK2-EOS-Bridge.git

Step 3 — Run the installer:

cd APC_MK2-EOS-Bridge

chmod +x APC-EOS_bridge_installer.sh

./APC-EOS_bridge_installer.sh

The Python script arrives from GitHub exactly as committed — no corruption possible. This is the recommended method for all future installs.

## 13.3 Updating Files

After making changes to any file on the Mac, push to GitHub:

cd ~/Documents

git add eos_apc_feedback.py eos_apc_config.json eos_apc_editor.html

git commit -m "Description of what changed"

git push

To add new files:

git add filename

git commit -m "Added filename"

git push

## 13.4 Pulling Updates on an Existing Mac

To update an existing install with the latest files from GitHub:

cd ~/Documents

git pull

After pulling an updated eos_apc_feedback.py, always reload the bridge: launchctl unload ~/Library/LaunchAgents/com.eos.apcfeedback.plist && launchctl load ~/Library/LaunchAgents/com.eos.apcfeedback.plist

## 13.5 Authentication

GitHub requires a Personal Access Token for push operations — your GitHub password does not work for Git commands.

To create a token:

- Go to github.com → Settings → Developer Settings → Personal Access Tokens → Tokens (classic)

- Generate new token (classic) — give it repo scope and No expiration

- Copy the token immediately — it is only shown once

When Git asks for a password, paste the token. To save it permanently in macOS Keychain:

git config --global credential.helper osxkeychain

After entering the token once it is stored and never asked for again.

## 13.6 Starting Fresh

If the repository gets into a broken state:

- On GitHub: Settings → scroll to bottom → Delete this repository → confirm

- Create a new empty repository — do NOT tick Add a README file

Then on the Mac:

cd ~/Documents

rm -rf .git

git init

git add eos_apc_feedback.py eos_apc_config.json eos_apc_editor.html

git commit -m "Initial commit"

git branch -M main

git remote add origin https://github.com/YOUR_USERNAME/APC_MK2-EOS-Bridge.git

git push -u origin main

End of Document